HEAT TRANSFER ENHANCEMENT OF WATER HEATING SYSTEM BY ADDING DIFFERENT MASS FRACTION OF AL2O3 NANO MATERIAL WITH RT-31 PCM

Dr. Vilas Hajare

Professor, Sharad Institute of Technology, College of Engineering, Yadrav-Ichalkaranji, Maharashtra-India

Abstract- Energy storage is most essential in industrial and commercial applications. In that thermal energy storage is need in thermal industries for heat and cold applications. Various researches are going on in the globe in the field of thermal storage and improving their performance for various applications. One of the methods to improve the performance of thermal storage is adding different phase change material with each other and adding nano particles with phase change materials. There is problem in stability of nanoparticle mixed thermal storage material to solve this problem oil soluble surfactant is used to coat the nanoparticles and to achieve the stability of nano added phase change materials. In this paper experimental work solar water heater is used as a thermal energy storage system and experiment were carried out with phase change material and Al2O3 combinations (0.50, 0.75, 1, and 1.25 %) to enhancement in heat transfer. These experiments are conducted by adding nanoparticles in different percentages into phase change material to enhance its thermal stability.

Keywords- Thermal storage, Latent heat, Performance improvement, Nanoparticles, encapsulations.

I  INTRODUCTION

Thermal energy storage is attractive technology for various applications like water heating, textile, cooking, wind and solar applications. Thermal storage maintains thermal stability between demand and supply during peak hour and store energy for longer duration of time. Thermal energy storage can be done by following three ways;   sensible storage, latent storage and thermochemical storage. Out of above latent thermal energy storage is mostly used for variety of renewable applications. In latent storage phase change of substance takes place. Latent energy storage can be done by using paraffin wax. Which changes its phase from sold to liquid and liquid to solid. The major challenge of using paraffin wax is low thermal conductivity and encapsulation techniques.  Most of the researchers are given focus on implementation of thermal energy storage by combinations of phase change material with different nanoparticles. Adding nanoparticles with various phase change material may improve the thermal stability and heat transfer.  The aim of this research is to implement latent thermal energy storage for solar water heating tank by adding Al2O3 nanoparticles with phase change material.

The performance of different types of heat exchangers used as latent heat thermal storage has been studied by many researchers, according to A. Valan Arasu et al. [1] thermal conductivity of paraffin wax can be enhanced by employing high conductivity materials such as alumina oxides. This paper given focus on numerical investigation has been carried out to find out the performance enhancement of paraffin wax with alumina oxide (Al2O3) particles in comparison with simple paraffin wax in a concentric double pipe heat exchanger application. S. Jegadheeswaran et al. [2] paper reviews influence of enhancement techniques on the thermal response of the PCM in terms of phase change rate and amount of latent heat stored/retrieved has been addressed. Also problems related mathematical modelling for enhancement of performance is also discussed in paper. Muath A. Alomair  et al. [3] The current study focused use of nanoparticle in phase change material to improve the thermal conductivity of the PCM. Experimentation is performed using CuO and Al2O3 nanoparticles with Sigma Aldrich and Rubitherm (RT-18) as base PCM, supplied by Rubitherm GmbH. The vertical cylindrical LHTES is composed of two concentric pipes; with the inner one carrying a heat transfer fluid at a constant temperature and the annular space containing a nano-PCM. The initial temperature of the nano- PCM is 5 0C while the temperature of the heat transfer fluid is 40 0C. The   results show that using nano-PCM reduces the melting time when compared to base PCM, but enhanced melting is observed when Copper Oxide nanoparticle is used. R. Meenakshi Reddy et al. [4]   experimentations are carried out in the TES system for different phase change materials (paraffin and Stearic acid) by varying HTF flow rates and for various sizes of spherical capsules (68, 58, and 38mm in diameter). Experiments are performed in both charging and discharging processes. The results show in (fig no.2) that, the 38mm diameter spherical capsule shows better performance compared to spherical capsules of other sizes.

Figure 1-2. Experimental set up and results for the different PCMs for the 38mm diameter capsule. [4]

Khaireldin Faraj [5] the study emphasizes the promising effectiveness of PCM in building heating applications, and highlights the significance of PCM system hybridization. The study shows that experimental investigations on commercial constructions, and hybrid systems development and optimization, are still required. B. Kanimozhi [6] Experiments were conducted in a fabricated thermal energy storage cylindrical tank embedded with copper tubes that in turn filled with these waxes in a liquid state.

Figure No. 3 (a) Cu tubes embedded in concentric circles and thermocouples arrangements, (b) filling of PCM, and (c) Cu tubes arrangements kept in a thermal energy storage chamber. [6]

II THERMAL STORAGE BY NANO-PCM

Due to thermal energy storage can be used to enhance energy efficiency and to improve energy conservation and management, studies on thermal energy storage materials have attracted increasing attention in recent years [7,8]. Such materials can be categorized into sensible heat storage materials [9], latent heat storage materials [10], and thermal chemical materials. Latent heat storage uses phase-change materials (PCMs) that have much higher heat storage density and extremely smaller temperature variation during a phase-change process, compared with sensible heat storage materials.

Figure No.  4 Classification of phase change materials

In comparison with thermal chemical materials and inorganic PCMs, organic PCMs have a proper phase-change temperature range, little or no super cooling, lower vapor pressure, no toxicity, non-corrosives, and excellent thermal stability. Therefore, organic PCMs are one of the most preferred forms for thermal energy storage and have promising applications [10, 11] in the field of solar energy storage, industrial waste energy storage, etc.

Figure No. 5 Properties of phase change materials (RT-31)

The shape-stability of PCM is always concerned when they are used in a thermal storage system. Therefore, the compressive strength of paraffin and PCM composites are detailed at different temperatures ranged from 25 to 90 ℃.The compressive strength of paraffin at room temperature is promoted step by step after first and second level packaging as concluded in (Fig. No. 3) the compressive strength of PCM increases from 3.01 to 6.25 MPa after infiltrating into the porous graphite foam. [12]

Figure No. 6 Compressive strength of paraffin and PCM composites at diff. temperature. [12]

Figure No. 7 Thermal Properties of different Nano fluids

It is well-known fact that metals NPs have thermal conductivities higher than those of fluids in solid form. For example, the thermal conductivity of copper at room temperature is about 700 times greater than that of water and about 3000 times greater than that of engine oil. Even oxides such as alumina (Al2O3) have thermal conductivity higher than that of water. Therefore, the fluids containing suspended solid particles are expected to display significantly enhanced thermal conductivities relative to those of conventional heat transfer fluids [13]. Nanofluids are produced by dispersing the nanometric scales solid particles into liquid such as water, ethylene glycol or oils. Nanofluids are expected to exhibit superior properties relative to those of conventional heat transfer fluids and fluids containing microscopic sized particles. Because the heat transfer takes place at the surface of the particles, it is desirable to use the particles with large total surface area. The large total surface area also increases the stability suspension (Lee et al., 1999) [14]. Recently it has been demonstrated that the nanofluids consisting of CuO or Al2O3 NPs in water or ethylene exhibit advance thermal conductivity (Cao, 2002) [15].

III EXPERIMENTATIONS AND RESULTS

Figure No. 8 Experimental test set up

Figure No. 9-10 Melting of nano-PCM and melting of pure PCM in various balls

Experimental test set up consist of well insulated hot water tank with test facility like wattmeter, temperature indicator, dimmerstat, rotameter and heater. The heat input (Ein), heat absorbed (Eab) and temperature profiles for charging and discharging are evaluated for the Al2O3 nanoparticles when combined with Paraffin phase change materials. For experimentation different mass fraction of Al2O3 (0.50, 0.75, 1, and 1.25 % Al2O3 with 3.5 kg of Paraffin wax (RT-31) are used. And comparisons are to be made with various mass fractions of nano phase change materials. During experimentation charging and dis charging of phase change material is done by latent heat principle. Through heater Nano-PCM bowls are heated and phase change of nano phase change material from solid to liquid takes place and during discharging process combination of nano phase change material dissipate energy and phase change occurs from sold to liquid. Results shows that implementing 0.50 % Al2O3 with phase change material gives 15% more heat transfer rate. Hence, combination of the nanoparticles with PCM gives better thermal performance during charging and discharging conditions. In the presence of a thermocouple, the solid PCM clasps itself to the thermocouple preventing the solid PCM from sinking/rising to the bottom/top to the capsule due to density difference of two phases of the PCM. It is observed that the melting of the PCM was concentric, and last point of melting was almost two thirds of the distance from the top. Initially, conduction was the dominant process. It created a constant liquid layer near the inner wall of the capsule. Similar phenomena were observed in the melting of different size capsules in this experiment.

IV RESULTS AND DISCUSSIONS

In first experiment pure PCM were kept at all the position. Melting behavior of each ball located at different position in the tank clearly visualized from the photographs shown in fig. 10. During observation it was clearly shown that time required for melting of B3 and B4 is less than the melting of ball B1 and B2. This happened due to the setting of natural current from hot end to the cold end of the tank. As time passes, temperature profile is set diagonally and the temperature difference at cold end increases than the hot end junction.

Figure No. 11 Melt fraction of nano mixed PCM ascending order

It is also found that the melting time reduces due to the addition of nanoparticles. This indicates enhancement in the heat transfer rate for nano mixed PCM. The PCM with 0.5%

Figure No. 12 Melt fraction of nano mixed PCM descending order

At starting condition effect of nanoparticles were not experienced. But nanoparticles assisted in heat transfer after growing of certain amount of melt zone. It is clearly shown in the fig. 9 that at mid-section there is abrupt increment in melt the mid-section there is abrupt increment in melt fraction with respect to time. This will noticeably shows the enhancement in the PCM by the addition of Al2O3 nanoparticles. In pure PCM experiment, there is difference in melting time for each ball is observed. But in this experiment at the end all the balls were melts nearly at same time. In pure PCM experiment ball B1 requires maximum time, but in this experiment its melting time reduces due to addition of Al2O3 nanoparticles. This observation obviously shows the enhancement in the pure PCM with addition of Al2O3 nanoparticles.

Conclusion

The TES system using the paraffin wax with nono particles as latent heat storage material all along one cycle (charging and discharging processes) was studied. Temperature distributions in the PCM and the inlet and outlet temperatures of the HTF were measured and recorded in the same way as in the melting period. Melting and solidification processes of the PCM were repeated at different initial HTF temperatures and mass flow rates.  The experiments performed showed that the enhancement in the heat transfer rate is depending on the percentage of nanoparticles and position of the material into the heat transfer fluid tank. From experimental investigation the enhancement is found in heat transfer rate of PCM due to the incorporation of nanoparticles. The enhancement is depends on the percentage of nanoparticles and position of the spherical ball into the tank. Maximum enhancement in melting time of 10.65% is observed for 0.5% Al2O3 when it is placed on B2 position in forward experiment. Experimental evidence indicates that effective thermal conductivity increases with decrease in particle size.

References

1. Valan, A.A., Sasmito, A.P. and Mujumdar, A.S., 2013. Numerical performance study of paraffin wax dispersed with alumina in a concentric pipe latent heat storage system. Thermal science, 17(2), pp.419-430.

2. Jegadheeswaran, S. and Pohekar, S.D., 2009. Performance enhancement in latent heat thermal storage system: a review. Renewable and Sustainable energy reviews, 13(9), pp.2225-2244.

3. Alomair, M.A., Alomair, Y.A., Abdullah, H.A., Mahmud, S. and Tasnim, S., 2017. Nanoparticle enhanced phase change material in latent heat thermal energy storage system: an experimental study. In Proc. Int. Conf. Energy Harvest. Storage, Transf (No. 119, pp. 1-5).

4. Reddy, R.M., Nallusamy, N. and Reddy, K.H., 2012. Experimental studies on phase change material-based thermal energy storage system for solar water heating applications. Journal of Fundamentals of Renewable Energy and Applications, 2.

5. Faraj, K., Khaled, M., Faraj, J., Hachem, F. and Castelain, C., 2020. Phase change material thermal energy storage systems for cooling applications in buildings: A review. Renewable and Sustainable Energy Reviews, 119, p.109579.

6. Kanimozhi, B., Bapu, B.R. and Pranesh, V., 2017. Thermal energy storage system operating with phase change materials for solar water heating applications: DOE modelling. Applied Thermal Engineering, 123, pp.614-624.

7. Yoo, D.W. and Joshi, Y.K., 2004. Energy efficient thermal management of electronic components using solid-liquid phase change materials. IEEE Transactions on Device and Materials Reliability, 4(4), pp.641-649.

8. Zalba, B., Marın, J.M., Cabeza, L.F. and Mehling, H., 2003. Review on thermal energy storage with phase change: materials, heat transfer analysis and applications. Applied thermal engineering, 23(3), pp.251-283.

9. F. Abniki, E. Bakhshi, M. Mosalla. The sensible heat storage by phase change materials, Petroleum Preprints Presented at the 235th ACS National Meeting, New Orleans, 6-10 April 2008, p. 110.

10. Agyenim, F., Hewitt, N., Eames, P. and Smyth, M., 2010. A review of materials, heat transfer and phase change problem formulation for latent heat thermal energy storage systems (LHTESS). Renewable and sustainable energy reviews, 14(2), pp.615-628.

11. Alawadhi, E.M., 2012. Using phase change materials in window shutter to reduce the solar heat gain. Energy and Buildings, 47, pp.421-429.

12. Tao, Z., Wang, H., Liu, J., Zhao, W., Liu, Z. and Guo, Q., 2017. Dual-level packaged phase change materials–thermal conductivity and mechanical properties. Solar Energy Materials and Solar Cells, 169, pp.222-225.

13. Khan, I., Saeed, K. and Khan, I., 2019. Nanoparticles: Properties, applications and toxicities. Arabian journal of chemistry, 12(7), pp.908-931.

14. Thomas, S. and Sobhan, C.B.P., 2011. A review of experimental investigations on thermal phenomena in nanofluids. Nanoscale research letters, 6(1), pp.1-21.

15. Brandow, S.L., Dressick, W.J., Dulcey, C.S., Koloski, T.S., Shirey, L.M., Schmidt, J. and Calvert, J.M., 1997. Nanolithography by displacement of catalytic metal clusters using an atomic force microscope tip. Journal of Vacuum Science & Technology B: Microelectronics and Nanometer Structures Processing, Measurement, and Phenomena, 15(5), pp.1818-1824.